I. Definition of display panel and key

Menu key: enter/return to the main menu

Up key: menu function select up/parameter increment

Down key: menu function select down/parameter decrement

Enter key: select the menu function, and confirm and save when modifying the menu function parameters.

II. Menu function

After the machine is powered on and reset, press the menu key to enter the main menu (exit after 30 seconds of no operation on the main menu interface). See the main menu function table for details (5 main menu options in total); Press the up or down key to select the main menu, press the enter key to modify the function parameters of each menu, and then press the enter key to save the current function parameters (with power-off memory after saving).

| Main menu | Submenu | Menu function description |  |
| --- | --- | --- | --- |
| 1. System settings | 1. DMX address | Modify the DMX address code up or down (the address code can be selected from 001-512), press the Enter key to save, and the default is 001. |  |
|  | 2. Encoder | Up or down to modify the encoder on/off, the confirmation key to save, the default on. |  |
|  | 3. X reversal | Modify X up or down to reverse on/off, enter to save, off by default. |  |
|  | 4. Y reversal | Modify Y up or down to reverse on/off, enter to save, off by default. |  |
| 2. Operation mode | 1. Operation mode | Up or down to select the operating mode DMX/Voice/Auto, enter to save, default to DMX. |  |
|  | 2. Channel selection | Select the channel CH18 up or down, and press CH42 to save. The default is CH18. |  |
| 3. Manual control | 1. X-axis | Modify the running parameter value of X motor upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 128. |  |
|  | 2. X rotation | Modify the X rotation parameter value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 3. X-axis fine adjustment | Modify up or down the X fine adjustment motor running parameter value (adjustable range 000-255), press the Enter key to save, and the default is 000. |  |
|  | 4. Y-axis | Modify the Y-axis parameter value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 5. Y rotation | Modify the Y rotation parameter value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 6. Y-axis fine adjustment | Modify the Y fine adjustment parameter value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 7. XY speed | Modify the XY speed parameter value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 8. Total dimming | Modify the total dimming value upward or downward (the adjustable range is 000-255). Press the Enter key to save. The default value is 255. |  |
|  | 9. Dimming | Modify the dimming value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 255. |  |
|  | 10. Strobe | Modify the stroboscopic value upward or downward (the adjustable range is 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 11. Red | Modify the brightness of the red lamp bead upward or downward (adjustable range: 000-255). Press the Enter key to save. The default value is 255. |  |
|  | 12. Green | Modify the brightness of the green lamp bead upward or downward (adjustable range: 000-255). Press the Enter key to save. The default value is 255. |  |
|  | 13. Blue | Modify the brightness of the blue lamp bead upward or downward (adjustable range: 000-255). Press the Enter key to save. The default value is 255. |  |
|  | 14. White | Modify the brightness of the white lamp bead upward or downward (adjustable range: 000-255), press the confirmation key to save, and the default value is 255. |  |
|  | 15. Mode | Modify the mode parameter value upward or downward (the adjustable range is 000-255), press the Enter key to save, and the default is 000. |  |
|  | 16. Speed | Modify the speed parameter value upward or downward (adjustable range: 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 17. Background color | Modify the parameter value of the background color upward or downward (the adjustable range is 000-255). Press the Enter key to save. The default value is 000. |  |
|  | 18. Background tone light | Modify the background tone light parameter value upward or downward (the adjustable range is 000-255), press the Enter key to save, and the default value is 000. |  |
|  | 19. Reset | Yes/No |  |
| 4. Display settings | 1. Language selection | Select Chinese/EN (English) up or down, and press Enter to save. The default is Chinese. |  |
|  | 2. Inverted display settings | Select up or down to close/open the reverse display setting, and press the confirmation key to save. It is closed by default. |  |
|  | 3. Screen saver | Select up or down to turn off/on the screen saver, press the enter key to save, and turn on by default. |  |
|  | 4. Software version | The G2FA program identification code cannot be modified. |  |
| 5. Factory settings | Display Please enter the password! Change up or down to 138, and then press the confirmation key to enter the factory setting. | 1. X-axis calibration | Modify the parameter up or down (range: 000-255), press the OK key to save, and the default value is 128. |
|  |  | 2. Y-axis calibration | Modify the parameter up or down (range: 000-255), press the OK key to save, and the default value is 128. |
|  |  | 3. Red | Modify the current of the red lamp bead up or down (032-255), press the OK key to save, and the default is 252. |
|  |  | 4. Green | Modify the current of the green lamp bead up or down (032-255), press the OK key to save, and the default is 252. |
|  |  | 5. Blue | Up or down to modify the blue lamp current (032-255), enter the key to save, the default is 252. |
|  |  | 6. White | Modify the current of the white lamp bead up or down (032-255), press the OK key to save, and the default is 252. |
|  |  | 7. Current | Select 000/007 up or down, press OK to save, and the default is 004. |
|  |  | 8. Speed | Select 000/007 up or down, press Enter to save, and the default is 000. |
|  |  | 9、Logo | Select 000/001 up or down and press OK to save. The default is 000. |
|  |  | 10. Temperature | Change the temperature parameter up or down (range 40-70), press the OK key to save, and the default value is 065. |
|  |  | 11. Temperature-Fan | Up or down to select off/on temperature-fan control, enter key to save, default on. |
|  |  | 12、LED | LED light bead sequence selection LEDA/LEDB |

III. Master-slave control

Two or more identical lamps are connected by DMX three-core signal line, all lamps are modified to any address code from 001 to 512, any one is set as the host, other lamps are slave, and the display screen of all slave does not flicker; When the host computer is used to adjust the effects of gradual change, pulse change, jump change, sound control and self-walking, all slave computers shall synchronize the effects of gradual change, pulse change, jump change, sound control and self-walking.

Special note: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash randomly and not be synchronized.

2. All lamps must work only when the DMX512 console is turned off.

IV. DMX512 control

After the address codes of all lamps are set, connect all lamps to the DMX512 console in parallel with the DMX three-core signal line. The address code stops flashing, indicating that the DMX512 console signal has been sent to the lamps. Control relevant functions with the DMX512console according to the channel description.

CH18 channel description:

| Passageway | Channel value | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | X-axis |
| 2 | 000-255 | The X-axis has no pole |
| 3 | 000-255 | X Fine tuning |
| 4 | 000-255 | Y-axis |
| 5 | 000-255 | The Y-axis has no pole |
| 6 | 000-255 | Y-axis fine adjustment |
| 7 | 000-255 | XY speed |
| 8 | 000-255 | Dimming |
| 9 | 000-255 | Strobe |
| 10 | 000-255 | Red |
| 11 | 000-255 | Green |
| 12 | 000-255 | Blue |
| 13 | 000-255 | White |
| 14 | 000-255 | Pattern |
| 15 | 000-255 | Speed |
| 16 | 000-255 | Background color |
| 17 | 000-255 | Background tonal light |
| 18 | 000-255 | Reset: When the parameter value is 150-255, the whole machine is reset. The console parameter value must be pulled below 10 and then pushed to 150-255 to be useful. When the parameter value is 000-149, it is useless and cannot be reset. |

CH42 channel description:

| Passageway | Channel value | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | X-axis |
| 2 | 000-255 | The X-axis has no pole |
| 3 | 000-255 | X Fine tuning |
| 4 | 000-255 | Y-axis |
| 5 | 000-255 | The Y-axis has no pole |
| 6 | 000-255 | Y-axis fine adjustment |
| 7 | 000-255 | XY speed |
| 8 | 000-255 | Dimming |
| 9 | 000-255 | Strobe |
| 10 | 000-255 | Red 1 |
| 11 | 000-255 | Green 1 |
| 12 | 000-255 | Blue 1 |
| 13 | 000-255 | White 1 |
| 14 | 000-255 | Red 2 |
| 15 | 000-255 | Green 2 |
| 16 | 000-255 | Blue 2 |
| 17 | 000-255 | White 2 |
| 18 | 000-255 | Red 3 |
| 19 | 000-255 | Green 3 |
| 20 | 000-255 | Blue 3 |
| 21 | 000-255 | White 3 |
| 22 | 000-255 | Red 4 |
| 23 | 000-255 | Green 4 |
| 24 | 000-255 | Blue 4 |
| 25 | 000-255 | White 4 |
| 26 | 000-255 | Red 5 |
| 27 | 000-255 | Green 5 |
| 28 | 000-255 | Blue 5 |
| 29 | 000-255 | White 5 |
| 30 | 000-255 | Red 6 |
| 31 | 000-255 | Green 6 |
| 32 | 000-255 | Blue 6 |
| 33 | 000-255 | White 6 |
| 34 | 000-255 | Red 7 |
| 35 | 000-255 | Green 7 |
| 36 | 000-255 | Blue 7 |
| 37 | 000-255 | White 7 |
| 38 | 000-255 | Pattern |
| 39 | 000-255 | Speed |
| 40 | 000-255 | Background color |
| 41 | 000-255 | Background tonal light |
| 42 | 000-255 | Reset: When the parameter value is 150-255, the whole machine is reset. The console parameter value must be pulled below 10 and then pushed to 150-255 to be useful. When the parameter value is 000-149, it is useless and cannot be reset. |

V. Technical parameters:

Voltage: AC100 ~ 240V 50/60HZ

Power: 180W

Lamp beads: 7 4-in-1 lamp beads

Control mode: DMX512, self-propelled, master-slave, voice control, with RDM function.

Channel: CH18, CH42

Dimming: 32bit 0 ~ 100% linear dimming

Features: XY head + beam + flash + XY stepless

Working temperature: -30 ℃ ~ 50 ℃

Strobe frequency: 1 ~ 30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP rating: IP20

Dimensions: Height: 23.5cm Length: 58 cm Width: 18 cm Weight: 5.9 kg